How to open a check account with MCR?

Clerks can open a check table with any type of magnetic card.

Please read the following instruction.

(The option number may be different for each local rom version

and some specific rom may not include this function as it already

had its own check table open with MCR function.)

1) Set P mode general function option "55. CARD READER ENABLE" as YES

2) Set P mode tracking option "15. CHECK OPEN WITH MCR" as YES

3) Choose which track# of MCR to use in P mode tracking option "19. CHECK OPEN WITH MCR"  *

4) Set P mode tracking option "17. MCR STARTING OFFSET"  **

5) Set P mode tracking option "18. MCR DATA LENGTH"  **

note)

* 	Most magnetic card has data written in track1,2 or track2,3. Please decide which track will you use

to open a check table. (You can choose either track2 or track3 here)

** 	Read magnetic card in "MCR" menu of S mode self test.

Now you can see the data written in magnetic card for each track.

Then you should make a decision where to start reading the check account number.

This is a "MCR STARTING OFFSET".

And now you also have to decide the length of an account number from the magnetic card.

For example, let's say that you get ";223027783445;875" from S mode self test MCR reading.

And let's assume that you set the "MCR STARTING OFFSET" as 6 digit and

"MCR DATA LENGTH" as 8 digit.

Now if you read the magnetic card in REG mode, SPS-2000 will open a check account with

account number "27783445".