SPS-2000 UPDATE INSTRUCTION

2006.06.27

SPS-2000 APPLICATION UPDATE FROM PC	2

SPS-2000 APPLICATION UPDATE FROM SD CARD	3

IMAGE DOWNLOAD FROM PC	4

IMAGE DOWNLOAD FROM SD CARD	5

OS UPDATE FROM PC	6

OS UPDATE FROM SD CARD	8

BOOTLOADER UPDATE FROM PC	9

BOOTLOADER UPDATE FROM SD CARD	11

# SPS-2000 APPLICATION UPDATE FROM PC

- Please download the sps2000Update.exe file from SAM4s ftp server.

- Please turn on the SPS-2000.

- Please set up your PC IP address with 192.168.0.2 and then set up subnet mask with 255.255.255.0

- Please execute the sps2000Update.exe file.

- Please connect SPS-2000 with pc by using cross cable. (LAN cable)

- Please set the TARGET REGISTER # as same as sps2000 register #.

- Please click the “Application Upgrade” and select the file named “sps2000”

# SPS-2000 APPLICATION UPDATE FROM SD CARD

- If you want to update application from SD card, please make a “sps2000” folder in SD card. And then copy the application image(sps2000) to this folder.

- Please insert SD card to sps2000.

- Press the “APPLICATION UPDATE” in H/W TEST menu.

- Confirm the update by pressing “YES” button.

# IMAGE DOWNLOAD FROM PC

- Please download the sps2000Update.exe file from SAM4s ftp server.

- Please turn on the SPS-2000.

- Please set up your PC IP address with 192.168.0.2 and then set up subnet mask with 255.255.255.0

- Please execute the sps2000Update.exe file.

- Please connect SPS-2000 with pc by using cross cable. (LAN cable)

- Please set the TARGET REGISTER # as same as sps2000 register #.

- Please click the “Picture Upload” and select the image.

- The length of image file name should be less than 20.

- The image size should be less than 60*50

- The image name should be alphabet only.

- SPS2000 is discriminate a capital letter and a small letter.

- You can use the downloaded images on KEY RELOCATION in S mode

TIP :

If you have image file named “LOGO.JPG” it will be used for screen saver logo.

# IMAGE DOWNLOAD FROM SD CARD

- The length of image file name should be less than 20.

- The image size should be less than 60*50

- The image name should be alphabet only.

- SPS2000 is discriminate from a capital letter and a small letter.

- SD card should be formatted with fat32.

- If you wish download the images from SD card, please make a “image” folder in SD card. And then please copy the images to this "image" folder.

- Please execute the “COPY” on “BUTTON IMAGE SELECTION” in

KEY RELOCATION then the images will be downloaded from

SD card to SPS2000

TIP :

If you have image file named “LOGO.JPG” it will be used for screen saver logo.

# OS UPDATE FROM PC

- Please unzip following files.
files : SolarWinds-TFTP-Server.zip, NandBoot.bin, zImage, ramdisk.gz, xlib.yaffs,
      app.yaffs

- Please install the program of SolarWinds-TFTP-Server.exe on your PC.

** Alternate TFTP server program is on SAM4S FTP server.

The name of the file is TFTPD32_300.ZIP.

If you have trouble with of SolarWinds-TFTP-Server, try this.

- Please set up your PC IP address as 192.168.0.2 and then set up subnet mask as 255.255.255.0

- Please execute SolarWinds-TFTP-Server program.

- Please select configuire in File menu.

- Please select Transmit and Receive files in Security tab.

- Please copy the files unzipped to c:\TFTP-Root\

- Please connect SPS-2000 with pc by using cross cable. (LAN cable)

- With the button(at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000. (For sure, with the button pushed, turn on the power.)

- Please input the system password. (password : 1287)

- Please click the “SYSTEM MENU”.

- Please click “Copy all” in the menu of SPS-2000.

- When you see failure message, please execute “Copy all” in the menu of SPS-2000 again. If you see failure message from the beginning, please check the procedure above from the beginning carefully. If you succeed the update, you can see “Image writing finished” message on the display. This procedure take 1 minute to 5 minute.

- After its updating, please shutdown the machine.

- With the button (at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000.

  - Please input the system password. (password : 1287)

  - Please click the “ALL CLEAR”.

# OS UPDATE FROM SD CARD

- Prepare following files.

files : FileDiv.exe, NandBoot.bin, zImage, ramdisk.gz, xlib.yaffs, app.yaffs

- Make a directory named “Update” on root of SD card.

- Copy following files to Update directory of SD card.

files : NandBoot.bin, zImage, ramdisk.gz, xlib.yaffs, app.yaffs

- Insert SD card to SPS-2000.

- With the button(at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000. (For sure, with the button pushed, turn on the power.)

- Please input the system password. (password : 1287)

- Please click the “SYSTEM MENU”.

- Please click “Copy all - SD” in the menu of SPS-2000.

- After its updating, please shutdown the machine.

- With the button (at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000.

Please input the system password. (password : 1287)

- Please click the “ALL CLEAR”.

# BOOTLOADER UPDATE FROM PC

- Prepare unzip following file.
files : BootRom.bin

- Please install the program of SolarWinds-TFTP-Server.exe on your PC.

** Alternate TFTP server program is on SAM4S FTP server.

The name of the file is TFTPD32_300.ZIP.

If you have trouble with of SolarWinds-TFTP-Server, try this.

- Please set up your PC IP address as 192.168.0.2 and then set up subnet mask as 255.255.255.0

- Please execute SolarWinds-TFTP-Server program.

- Please select configuire in File menu.

- Please select Transmit and Receive files in Security tab.

- Please copy the files unzipped to c:\TFTP-Root\

- Please connect SPS-2000 with pc by using cross cable. (LAN cable)

- With the button(at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000. (For sure, with the button pushed, turn on the power.)

- Please input the system password. (password : 1287)

- Please click the “SYSTEM MENU”.

- Please click “Copy all” in the menu of SPS-2000.

- When you see failure message, please execute “BOOT UPDATE” in the menu of SPS-2000 again. If you see failure message from the beginning, please check the procedure above from the beginning carefully. If you succeed the update, you can see “Image writing finished” message on the display. This procedure take 1 minute to 5 minute.

- After its updating, please shutdown the machine.

# BOOTLOADER UPDATE FROM SD CARD

- Prepare following files.

files : BootRom.bin

- Make a directory named “Update” on root of SD card.

- Copy following files to “Update” directory of SD card.

files : BootRom.bin

- Insert SD card to SPS-2000.

- With the button(at the right botton at the front of LCD) pushed, please turn on the power of SPS-2000. (For sure, with the button pushed, turn on the power.)

- Please input the system password. (password : 1287)

- Please click the “SYSTEM MENU”.

- Please click “BOOT UPDATE - SD” in the menu of SPS-2000.

- After its updating, please shutdown the machine.