SPS-500 Memory All Clear

Complete clearing of all memory areas and installation of the default program can be done through the following special procedure:

- Turn the power switch located on the right side of the register to the OFF position.

- Turn the control lock to the unmarked position one position clockwise from the PGM position.

- Press and hold the key position where the CHECK key is located on the default keyboard layout.

TO MEMORY ALL CLEAR:

Press and hold this key position during power-up in S-mode.

- While continuing to hold the CHECK key, turn the power switch to the ON position.

- Press the upper left key of the keyboard, then the lower left key, then the upper right key, and finally press the lower right key.

- When complete, the display will prompt “Enter Register Number (1-32)”. Type the register number on the numeric keypad and press ENTER. The S-MODE PROGRAMMING MENU will display.

Entering Boot menu of SPS-500

You can enter the boot menu to setup and/or update the register through the following special procedure:

- Turn the power switch located on the right side of the register to the OFF position.

- Turn the control lock to the unmarked position one position clockwise from the PGM position.

- Press and hold the key position where the CASH key is located on the default keyboard layout.

TO Enter boot menu:

Press and hold this key position during power-up in S-mode.

- While continuing to hold the CASH key, turn the power switch to the ON position.

- Press the upper left key of the keyboard, then the lower left key, then the upper right key, and finally press the lower right key.

- Boot menu will appear.