How to use bitmap logo on a internal printer of SPS500

1) Copy images to SD card

You can update logo images to SPS500 by SD card.

-File name : logo_01.bmp ~ logo_15.bmp (Max. 15 logo images)

-Folder : Image logo files should stay in SD:\update\sps500 folder.

-File format : Black and white bitmap file(.bmp)

-File specification

a) SPS530 model : Up to 576 x 240 pixel bmp file.

b) SPS520 model : Up to 384 x 240 pixel bmp file.

2) Update images files from SD to SPS500

a) Place the mode lock to S mode.

b) Power on while holding down the CASH key(right bottom key in real keyboard) until you hear

the buzzer beep.

c) Press left top key, left bottom key, right top key and right bottom key to get into BOOT menu.

d) Enter boot menu password (1287)

e) Choose “Setup” -> “Logo Define” menu.

f) Insert SD card with logo images then touch the screen to start copying images from SD card to

SPS500.